Digital Hardware Design & Security

RTL Design, Fault Injection & Co-Simulation

# Part I: RTL Design Fundamentals

## 1. What is RTL?

Register Transfer Level (RTL) describes how digital circuits move and process data using registers and logic on each clock tick. Think of it as a factory assembly line: registers are workstations holding parts, combinational logic is machinery transforming those parts, and the clock is the timing bell synchronizing everything.

### Key Components

- Registers: Storage elements updating on clock edges (e.g., 8-bit register holds 0-255)

- Combinational Logic: Outputs depend only on current inputs (adders, multiplexers, comparators)

- Clock & Reset: Periodic signal defining circuit rhythm; reset initializes to known state

- Datapath: Holds/transforms data (registers, ALUs, shifters, buses)

- Controller (FSM): Sends control signals deciding when to load, compute, or output

### Design Flow

Write RTL (Verilog/VHDL) → Simulate with testbenches → Synthesize to gates → Place-and-route → Generate bitstream/layout

## 2. Verilog Examples

### 8-Bit Register

always @(posedge clk) begin

if (rst) q <= 8'd0; else q <= d_in;

end

### 4-Bit Counter

always @(posedge clk) begin

if (rst) count <= 0; else count <= count + 1;

end

### FSM (Traffic Light)

always @(posedge clk)

state <= rst ? RED : next_state;

always @*

case(state) RED:next=GREEN; GREEN:next=YELLOW;

YELLOW:next=RED; endcase

## 3. Serial Communication Protocols

- UART: Asynchronous, 2-wire (TX/RX). Frame: start bit + 5-9 data bits + parity + stop bits. Baud rate generator + TX/RX FSMs

- SPI: Synchronous, 4-wire (SCK, MOSI, MISO, CS). Full-duplex. Master generates clock, shift register handles data

- I²C: Synchronous, 2-wire (SCL, SDA). Multi-master. Start/stop conditions, address-based, ACK/NACK protocol

# Part II: Hardware Security & Fault Injection

## 4. Introduction to Fault Injection

Fault injection simulates attacks by deliberately causing errors to test system behavior. Like crash test dummies for hardware security—we test defenses before devices reach customers.

### Key Technologies

- RTL: Hardware blueprint in Verilog/VHDL

- QEMU: Virtual machine for software emulation

- Verilator: Converts RTL to fast C++ simulation

- Co-Simulation: Runs hardware (RTL) and software (QEMU) together via communication bridge

## 5. Types of Fault Attacks

### Voltage Glitching

Temporarily alters power supply causing computation errors or instruction skips. Like flickering lights—electronics malfunction briefly.

if (password == correct) grant_access=1; // Glitch→skips check

### Bit Flip Attacks

Flips single bit (0→1 or 1→0) in memory/registers. Example: user_privilege = 0 (normal) → 1 (admin).

### Clock Glitching

Manipulates clock timing. Like speeding up a metronome—chips skip instructions.

step1:verify(); step2:check(); step3:execute(); // May skip step2

## 6. Co-Simulation Workflow

### Step 1: Design RTL with Fault Input

module secure_module(clk, rst, fault_inject, key, access);

always @(posedge clk) begin

if (fault_inject) key <= 128'hDEADBEEF; // Corrupt

else if (input_key==key) access <= 1;

end

endmodule

### Step 2: Simulate with Verilator

verilator --cc module.v --exe testbench.cpp

make -C obj_dir -f Vmodule.mk && ./obj_dir/Vmodule

### Step 3: QEMU Attack Software

#define FAULT_ADDR 0x40000010

*(volatile uint32_t*)FAULT_ADDR = 1; // Trigger fault

status = *(volatile uint32_t*)STATUS_ADDR; // Read result

### Step 4: Communication Bridge

void qemu_hw_write(addr, val) {

if(addr==0x40000010) rtl->fault_inject = val & 1;

}

uint64_t qemu_hw_read(addr) {

if(addr==0x40000020) return rtl->access_granted;

}

## 7. Complete Attack Example

### Vulnerable Password Checker

always @(posedge clk) begin

if(fault_en) access<=1; // Skip check!

else if(pwd==CORRECT) access<=1; else access<=0;

end

### Attack Code

// Attempt 1: No fault → Result: 0 (Denied)

set_password(WRONG); enable_check();

// Attempt 2: With fault → Result: 1 (Granted!)

enable_fault(); set_password(WRONG); enable_check();

## 8. Countermeasures

### Triple Redundancy with Majority Voting

reg c1,c2,c3;

always @(posedge clk) begin

c1 <= (pwd==CORRECT); c2 <= (pwd==CORRECT);

c3 <= (pwd==CORRECT);

access <= (c1&c2)|(c2&c3)|(c1&c3); // Majority

end

### Other Techniques

- Temporal Redundancy: Repeat operations at different times

- Error Detection Codes: Checksums, parity, cryptographic MACs

- Randomization: Randomize timing/locations to hinder precise attacks

- Sensors: Voltage, temperature, clock monitors detect anomalies

# Part III: Practical Implementation

## 9. Tool Setup

| Purpose | Tool | Installation |
| --- | --- | --- |
| Virtual HW | QEMU | apt install qemu-system |
| RTL Sim | Verilator | apt install verilator |
| Compilation | GCC | apt install build-essential |
| Debug/View | GDB/GTKWave | apt install gdb gtkwave |

## 10. Learning Path

### Weeks 1-4: Beginner

Build simple password checker → Simulate with Verilator → Write fault injection testbench → Document security failures

### Weeks 5-12: Intermediate

Set up QEMU custom device → Bridge QEMU-Verilator → Implement voltage glitch simulation → Test against AES module → Design countermeasures

### Weeks 13+: Advanced

Build complete SoC with CPU → Implement secure boot with fault resistance → Conduct power analysis + fault injection → Write research paper → Contribute to open-source security tools

## 11. Common Pitfalls & Solutions

- Timing Issues: QEMU/RTL run at different speeds. Solution: Use sync signals, wait for ready states

- Memory Mapping: Address mismatch. Solution: Create address translation layer in bridge

- Incomplete Sensitivity: Use always @* for combinational blocks

- Unintended Latches: Provide default values, cover all cases

## 12. Resources

### Online Platforms

HDLBits (hdlbits.01xz.net) • EDA Playground (edaplayground.com) • CryptoHack • HackTheBox • IEEE CSAW ESC

### Documentation

Verilator (verilator.org) • QEMU (qemu.org/docs) • ChipWhisperer toolkit • Riscure training

### FPGA Boards

Basys 3 (entry-level Xilinx) • Nexys A7 / Arty A7 (advanced) • DE10-Lite (Intel)

### Key Papers

"Fault Attacks on Cryptographic Devices" (Boneh) • "Clock Glitch Attacks" (Yuce) • "Electromagnetic Fault Injection" (Dehbaoui)

## 13. Real-World Applications

- Virtual SoC: Complete chip systems, CPU-memory-peripheral interactions

- Driver Testing: Linux device drivers before hardware exists

- Secure Enclaves: TEE development, attack testing

- Research: Publish at CHES, HOST, IEEE S&P conferences

- Careers: AMD, Intel, ARM, Apple, NVIDIA, security consulting

# Key Takeaways

- RTL describes hardware at register/logic level—the bridge between code and silicon

- Three clocked steps: read registers → compute → store on clock edge (parallel execution)

- Datapath holds/transforms data; controller FSM orchestrates operations

- Fault injection simulates voltage glitches, bit flips, clock glitches to test security

- Co-simulation bridges RTL (Verilator) and software (QEMU) for complete system testing

- Countermeasures: redundancy, majority voting, temporal checks, error codes, sensors

- Tools: QEMU + Verilator + GCC + GDB/GTKWave (free and powerful)

- Practice on HDLBits, EDA Playground; deploy to FPGA boards; contribute to research

⚠ Use these skills defensively to build secure systems ⚠

Never attack real devices without authorization